| Kalam Telecom IP Addresses |  |
| --- | --- |
| Internal Network | 172.17.0.0/16 |
| MPLS backbone | 192.168.1.0/24 |
| Public IP | 74.211.4.0/27 |

| Kalam Telecom VLANs |  |
| --- | --- |
| IT Department | 172.17.10.0/24 |
| Finance Department | 172.17.20.0/24 |
| SWManagement VLAN | 172.17.99.0/24 |
| Infrastructure Devices | 172.17.100.0/24 |

| Batelco IP Addresses |  |
| --- | --- |
| Public IP | 54.40.61.0/25 |

| ABC Company Branch 1 VLANs |  |
| --- | --- |
| IT Department | 172.20.10.0/24 |
| Sales Department | 172.20.20.0/24 |
| Executive VLAN | 172.20.30.0/24 |

| ABC Company Branch 2 VLANs |  |
| --- | --- |
| IT Department | 172.21.10.0/24 |
| Accounting Department | 172.21.20.0/24 |
| Executive VLAN | 172.21.30.0/24 |

| XYZ Company Branch 1 VLANs |  |
| --- | --- |
| IT Department | 172.23.10.0/24 |
| Sales Department | 172.23.20.0/24 |
| Executive VLAN | 172.23.30.0/24 |

| XYZ Company Branch 2 VLANs |  |
| --- | --- |
| IT Department | 172.24.10.0/24 |
| Accounting Department | 172.24.20.0/24 |
| Executive VLAN | 172.24.30.0/24 |

| XYZ Company Branch 3 VLANs |  |
| --- | --- |
| IT Department | 172.25.10.0/24 |
| Accounting Department | 172.25.20.0/24 |
| Executive VLAN | 172.25.30.0/24 |

| Kalam Telecom IP Address Shema |  |  |  |  |
| --- | --- | --- | --- | --- |
| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| Kalam-PE1 | E0/0 (Kalam-PE2) | 192.168.1.1 | 255.255.255.252 | N/A |
|  | E0/1 (Kalam-PE3) | 192.168.1.5 | 255.255.255.252 | N/A |
|  | S1/0 (XYZ-1-CE) | 74.211.4.5 | 255.255.255.252 | N/A |
|  | S1/1 (ABC-1-CE) | 74.211.4.1 | 255.255.255.252 | N/A |
|  | Lo0 | 1.1.1.1 | 255.255.255.255 | N/A |
|  | Lo1 | 11.11.11.1 | 255.255.255.255 | N/A |
| Kalam-PE2 | E0/0 (Kalam-PE1) | 192.168.1.2 | 255.255.255.252 | N/A |
|  | E0/1 (Kalam-P2) | 192.168.1.9 | 255.255.255.252 | N/A |
|  | E0/2 (Kalam-R1) | 172.17.255.2 | 255.255.255.252 | N/A |
|  | S1/0 (Batelco-PE1) | 74.211.4.9 | 255.255.255.252 | N/A |
|  | S1/1 (XYZ-1-CE) | 74.211.4.13 | 255.255.255.252 | N/A |
|  | Lo0 | 1.1.1.2 | 255.255.255.255 | N/A |
|  | Lo1 | 11.11.11.2 | 255.255.255.255 | N/A |
| Kalam-PE3 | E0/0 (Kalam-P1) | 192.168.1.13 | 255.255.255.252 | N/A |
|  | E0/1 (Kalam-PE1) | 192.168.1.6 | 255.255.255.252 | N/A |
|  | E0/2 (Kalam-R1) | 172.17.255.6 | 255.255.255.252 | N/A |
|  | S1/0 (ABC-1-CE) | 74.211.4.17 | 255.255.255.252 | N/A |
|  | Lo0 | 1.1.1.3 | 255.255.255.255 | N/A |
|  | Lo1 | 11.11.11.3 | 255.255.255.255 | N/A |
| Kalam-PE4 | E0/0 (Kalam-PE6) | 192.168.1.38 | 255.255.255.252 | N/A |
|  | E0/1 (Kalam-PE5) | 192.168.1.33 | 255.255.255.252 | N/A |
|  | S1/0 (XYZ-2-CE) | 74.211.4.21 | 255.255.255.252 | N/A |
|  | S1/1 (XYZ-2-CE) | 74.211.4.25 | 255.255.255.252 | N/A |
|  | Lo0 | 1.1.1.4 | 255.255.255.255 | N/A |
|  | Lo1 | 11.11.11.4 | 255.255.255.255 | N/A |
| Kalam-PE5 | E0/0 (Kalam-P3) | 192.168.1.30 | 255.255.255.252 | N/A |
|  | E0/1 (Kalam-PE4) | 192.168.1.34 | 255.255.255.252 | N/A |
|  | E0/2 (Kalam-R2) | 172.17.255.14 | 255.255.255.252 | N/A |
|  | S1/0 (ABC-2-CE) | 74.211.4.29 | 255.255.255.252 | N/A |
|  | Lo0 | 1.1.1.5 | 255.255.255.255 | N/A |
|  | Lo1 | 11.11.11.5 | 255.255.255.255 | N/A |
| Kalam-PE6 | E0/0 (Kalam-PE4) | 192.168.1.37 | 255.255.255.252 | N/A |
|  | E0/1 (Kalam-P4) | 192.168.1.26 | 255.255.255.252 | N/A |
|  | E0/2 (Kalam-R2) | 172.17.255.10 | 255.255.255.252 | N/A |
|  | S1/1 (XYZ-2-CE) | 74.211.4.33 | 255.255.255.252 | N/A |
|  | Lo0 | 1.1.1.6 | 255.255.255.255 | N/A |
|  | Lo1 | 11.11.11.6 | 255.255.255.255 | N/A |
| Kalam-P1 | E0/0 (Kalam-PE3) | 192.168.1.14 | 255.255.255.252 | N/A |
|  | E0/1 (Kalam-P3) | 192.168.1.21 | 255.255.255.252 | N/A |
|  | Lo0 | 1.1.2.1 | 255.255.255.255 | N/A |
| Kalam-P2 | E0/0 (Kalam-P4) | 192.168.1.17 | 255.255.255.252 | N/A |
|  | E0/1 (Kalam-PE2) | 192.168.1.10 | 255.255.255.252 | N/A |
|  | Lo0 | 1.1.2.2 | 255.255.255.255 | N/A |
| Kalam-P3 | E0/0 (Kalam-PE5) | 192.168.1.29 | 255.255.255.252 | N/A |
|  | E0/1 (Kalam-P1) | 192.168.1.22 | 255.255.255.252 | N/A |
|  | Lo0 | 1.1.2.3 | 255.255.255.255 | N/A |
| Kalam-P4 | E0/0 (Kalam-P2) | 192.168.1.18 | 255.255.255.252 | N/A |
|  | E0/1 (Kalam-PE6) | 192.168.1.25 | 255.255.255.252 | N/A |
|  | Lo0 | 1.1.2.4 | 255.255.255.255 | N/A |
| Kalam-R1 | E0/0 (Kalam-PE2) | 172.17.255.1 | 255.255.255.252 | N/A |
|  | E0/1 (KKalam-PE3) | 172.17.255.5 | 255.255.255.252 | N/A |
|  | E0/2.10 | 172.17.10.1 | 255.255.255.0 | N/A |
|  | E0/2.20 | 172.17.20.1 | 255.255.255.0 | N/A |
|  | E0/2.99 | 172.17.99.1 | 255.255.255.0 | N/A |
|  | E0/2.100 | 172.17.100.1 | 255.255.255.0 | N/A |
|  | E0/3 (Kalam-R2) | 172.17.255.18 | 255.255.255.252 | N/A |
| Kalam-R2 | E0/0 (Kalam-PE6) | 172.17.255.9 | 255.255.255.252 | N/A |
|  | E0/1 (Kalam-PE5) | 172.17.255.13 | 255.255.255.252 | N/A |
|  | E0/2.10 | 172.17.10.2 | 255.255.255.0 | N/A |
|  | E0/2.20 | 172.17.20.2 | 255.255.255.0 | N/A |
|  | E0/2.99 | 172.17.99.2 | 255.255.255.0 | N/A |
|  | E0/2.100 | 172.17.100.2 | 255.255.255.0 | N/A |
|  | E0/3 (Kalam-R1) | 172.17.255.17 | 255.255.255.252 | N/A |
| Kalam-SW1 | VLAN 99 | 172.17.99.11 | 255.255.255.0 | 172.17.99.100 |
| Kalam-SW2 | VLAN 99 | 172.17.99.12 | 255.255.255.0 | 172.17.99.100 |
| Kalam-SW3 | VLAN 99 | 172.17.99.13 | 255.255.255.0 | 172.17.99.100 |
| IT-Desktop | E0 | 172.17.10.10 | 255.255.255.0 | 172.17.10.100 |
| Finance-Desktop | E0 | 172.17.20.10 | 255.255.255.0 | 172.17.20.100 |
| AAA-Syslog Server | E0 | 172.17.100.10 | 255.255.255.0 | 172.17.100.100 |

| Batelco IP Address Schema |  |  |  |  |
| --- | --- | --- | --- | --- |
| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| Batelco-PE1 | E0/0 (Batelco-PE2) | 54.40.61.1 | 255.255.255.252 | N/A |
|  | E0/1 (Batelco-PE3) | 54.40.61.5 | 255.255.255.252 | N/A |
|  | S1/0 (Kalam-PE1) | 74.211.4.10 | 255.255.255.252 | N/A |
|  | Lo0 | 2.2.2.1 | 255.255.255.255 | N/A |
| Batelco-PE2 | E0/0 (Batelco-PE1) | 54.40.61.2 | 255.255.255.252 | N/A |
|  | E0/1 (Batelco-PE4) | 54.40.61.9 | 255.255.255.252 | N/A |
|  | Lo0 | 2.2.2.2 | 255.255.255.255 | N/A |
| Batelco-PE3 | E0/0 (Batelco-PE4) | 54.40.61.13 | 255.255.255.252 | N/A |
|  | E0/1 (Batelco-PE1) | 54.40.61.6 | 255.255.255.252 | N/A |
|  | Lo0 | 2.2.2.3 | 255.255.255.255 | N/A |
| Batelco-PE4 | E0/0 (Batelco-PE3) | 54.40.61.14 | 255.255.255.252 | N/A |
|  | E0/1 (Batelco-PE2) | 54.40.61.10 | 255.255.255.252 | N/A |
|  | S1/0 (XYZ-3-CE) | 54.40.61.17 | 255.255.255.252 | N/A |
|  | Lo0 | 2.2.2.4 | 255.255.255.255 | N/A |

| ABC Company Branch 1 (172.20.0.0/16) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| ABC-1-CE | S1/0 (Kalam-PE3) | 74.211.4.18 | 255.255.255.252 | N/A |
|  | S1/1 (Kalam-PE1) | 74.211.4.2 | 255.255.255.252 | N/A |
|  | E0/0.10 | 172.20.10.1 | 255.255.255.0 | N/A |
|  | E0/0.20 | 172.20.20.1 | 255.255.255.0 | N/A |
|  | E0/0.30 | 172.20.99.1 | 255.255.255.0 | N/A |
|  | Lo0 | 10.10.10.10 | 255.255.255.255 | N/A |
| ABC-1-SW | VLAN 99 | 172.20.99.2 | 255.255.255.0 | 172.20.99.1 |
| Desktop | E0 | 172.20.10.10 | 255.255.255.0 | 172.20.10.1 |

| ABC Company Branch 2 (172.21.0.0/16) |  |  |  |  |
| --- | --- | --- | --- | --- |
| ABC-2-CE | S1/0 (Kalam -PE5) | 74.211.4.30 | 255.255.255.252 | N/A |
|  | S1/1 (Kalam -PE4) | 74.211.4.26 | 255.255.255.252 | N/A |
|  | E0/0.10 | 172.21.10.1 | 255.255.255.0 | N/A |
|  | E0/0.20 | 172.21.20.1 | 255.255.255.0 | N/A |
|  | E0/0.30 | 172.21.99.1 | 255.255.255.0 | N/A |
|  | Lo0 | 20.20.20.20 | 255.255.255.255 | N/A |
| ABC-2-SW | VLAN 99 | 172.21.99.2 | 255.255.255.0 | 172.21.99.1 |
| Desktop | E0 | 172.21.10.10 | 255.255.255.0 | 172.21.10.1 |

| XYZ Company Branch 1 (172.23.0.0/16) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| XYZ-1-CE | S1/0 (Kalam-PE1) | 74.211.4.6 | 255.255.255.252 | N/A |
|  | S1/1 (Kalam-PE2) | 74.211.4.14 | 255.255.255.252 | N/A |
|  | E0/0.10 | 172.23.10.1 | 255.255.255.0 | N/A |
|  | E0/0.20 | 172.23.20.1 | 255.255.255.0 | N/A |
|  | E0/0.30 | 172.23.99.1 | 255.255.255.0 | N/A |
|  | Lo0 | 110.110.110 | 255.255.255.255 | N/A |
| XYZ-1-SW | VLAN 99 | 172.23.99.2 | 255.255.255.0 | 172.23.99.1 |
| Desktop | E0 | 172.23.10.10 | 255.255.255.0 | 172.23.10.1 |

| XYZ Company Branch 2 (172.24.0.0/16) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| XYZ-2-CE | S1/0 (Batelco-PE4) | 74.211.4.22 | 255.255.255.252 |  | N/A |
|  | S1/1 (Batelco-PE6) | 74.211.4.34 | 255.255.255.252 |  | N/A |
|  | E0/0.10 | 172.24.10.1 | 255.255.255.0 |  | N/A |
|  | E0/0.20 | 172.24.20.1 | 255.255.255.0 |  | N/A |
|  | E0/0.30 | 172.24.99.1 | 255.255.255.0 |  | N/A |
|  | Lo0 | 120.120.120.120 | 255.255.255.255 |  | N/A |
| XYZ-2-SW | VLAN 99 | 172.24.99.2 | 255.255.255.0 |  | 172.24.99.1 |
| Desktop | E0 | 172.24.10.10 |  | 255.255.255.0 | 172.24.10.1 |

| XYZ Company Branch 3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| XYZ-3-CE | S1/0 (Batelco-PE4) | 54.40.61.18 | 255.255.255.252 | N/A |
|  | E0/0.10 | 172.25.10.1 | 255.255.255.0 | N/A |
|  | E0/0.20 | 172.25.20.1 | 255.255.255.0 | N/A |
|  | E0/0.30 | 172.25.30.1 | 255.255.255.0 | N/A |
|  | Lo0 | 130.130.130.130 | 255.255.255.255 | N/A |
| XYZ-3-SW | VLAN 99 | 172.25.99.2 | 255.255.255.0 | 172.25.99.1 |
| Desktop | E0 | 172.25.10.10 | 255.255.255.0 | 172.25.10.1 |

| VRF IDs |  |
| --- | --- |
| ABC company | 100 |
| XYZ Company | 200 |

| Route Distinguisher (RD) Table |  |  |
| --- | --- | --- |
| VRF | Route Distinguisher | Router / Interface |
| ABC company | 1.1.1.1:100 | Kalam-PE1 / S1/1 |
|  | 1.1.1.3:100 | Kalam-PE3 / S1/0 |
|  | 1.1.1.4:100 | Kalam -PE4 / S1/1 |
|  | 1.1.1.5:100 | Kalam -PE5 / S1/0 |
| XYZ Company | 1.1.1.1:200 | Kalam -PE1 / S1/0 |
|  | 1.1.1.2:200 | Kalam -PE2 / S1/1 |
|  | 1.1.1.4:200 | Kalam -PE4 / S1/0 |
|  | 1.1.1.6:200 | Kalam -PE6/ S1/1 |
|  | 2.2.2.4:200 | Batelco-PE4 / S1/0 |

| Route Target (RT) Table |  |
| --- | --- |
| VRF | Route Target |
| ABC company | 65001:100 |
| XYZ Company | 65002:200 |

| VTP |  |
| --- | --- |
| VTP Domain Name | KalamVTP |
| VTP Password | 123@Kalam |

| Usernames & Passwords |  |  |
| --- | --- | --- |
| Username | Password | Device |
| LocalAdmin | 123456 | Local Device |
| RadAdmin | 654321 | Radius Server |

| Router IDs |  |
| --- | --- |
| Kalam-PE1 | 1.1.1.1 |
| Kalam-PE2 | 1.1.1.2 |
| Kalam-PE3 | 1.1.1.3 |
| Kalam-PE4 | 1.1.1.4 |
| Kalam-PE5 | 1.1.1.5 |
| Kalam-PE6 | 1.1.1.6 |
| Kalam-P1 | 1.1.2.1 |
| Kalam-P2 | 1.1.2.2 |
| Kalam-P3 | 1.1.2.3 |
| Kalam-P4 | 1.1.2.4 |
| Kalam-R1 | 1.1.3.1 |
| Kalam-R2 | 1.1.3.2 |
| Batelco-PE1 | 2.2.2.1 |
| Batelco-PE2 | 2.2.2.2 |
| Batelco-PE3 | 2.2.2.3 |
| Batelco-PE4 | 2.2.2.4 |